|  | Technical Handbooks of FRM4VEG Instrumentation (TR-1): Headwall UAV version 1.0 National Physical Laboratory University of Southampton EOLAB 28 May 2020 |
| --- | --- |
|  | This document was produced as part of the ESA-funded project “Fiducial Reference Measurements for Vegetation Phase 2 (FRM4VEG 2)” under ESA contract number: 4000129823/20/I-NS |

##### Authors

|  | Name | Organisation | Date |
| --- | --- | --- | --- |
| Written by | Niall Origo | NPL | 28/05/2020 |
|  | Rosalinda Morrone | NPL |  |
|  | Joanne Nightingale | NPL |  |
| Reviewed by (consortium) |  |  |  |
| Approved by (ESA) |  |  |  |

##### Version History

| Version | Date | Publicly available or private to consortium |
| --- | --- | --- |
| 1.0 | 28/05/2020 | Private Consortium |

##### Table of Contents

Authors	1

Version History	1

Table of Contents	2

List of figures	3

List of tables	3

Acronyms	3

1	Introduction	4

1.2	Purpose and Scope	4

2	Technical Description	5

2.1	Overview	5

2.2	Theory of Operation	9

3	Calibration History and Uncertainty Budget	11

3.1	Calibration History	11

3.2	Uncertainty Budget	11

4	Instrument Operation	11

4.1	Instrument Setup	11

4.2	Performing a Measurement	11

5	Care and Storage	12

A	Appendix	13

A.1	Calibration Report	13

6	Applicable and Reference Documents	13

# List of figures

Figure 1: Headwall VNIW-SWIR Co-Aligned + Internal Data Storage + GPS/IMU + LiDAR [1]	9

Figure 2: Trimble AP15 GNSS-inertial system circuit board [3]	10

Figure 3: Puck Lite Velodyne LiDAR [2]	11

# List of tables

Table 1: Technical characteristics of the Hyperspectral Co-Aligned VNIR/SWIR Airborne Sensor with DJI Matrice 600 Pro [1]	6

Table 2: Technical characteristics of Light weight Real-Time 3D LiDAR Sensor [2]	6

Table 3: Technical characteristics of the Trimble AP15 GNSS-Inertial System [3]	7

# Acronyms

| Abbreviation | Stands For |
| --- | --- |
| CMOS | Complementary metal-oxide semiconductor |
| DEM | Digital Elevation Model |
| ESA | European Space Agency |
| FRM4VEG | Fiducial Reference Measurements for Vegetation |
| GNSS | Global Navigation Satellite System |
| GPS | Global positioning system |
| IMU | Inertial Measurement Unit |
| LIDAR | Light detection and ranging |
| MEMS | Micro Electro-Mechanical Systems |
| MCT | Mercury Cadmium Telluride |
| OEM | Original Equipment Manufacturer |
| SWIR | Short Wave Infrared |
| UAV | Unmanned Aerial Vehicle |
| VNIR | Visible and Near Infrared |

# Introduction

## Purpose and Scope

This document forms part of deliverable D-60 of the European Space Agency (ESA) project ‘Fiducial Reference Measurements for Vegetation (FRM4VEG)’ and it should be used as a guide to operate the Headwall UAV. Its purpose is to provide an instrument technical description, together with information about maintenance and calibration history, pre-deployment uncertainties estimates, and steps required to achieve the FRM status.

The document is organized into …. key sections:

- Section 1 provides a summary of the document.

- Section 2 overviews the technical characteristics of the UAV together with a description of its functioning.

- Section 3 …..

- Section 4 ….

- Section 5 ….

- Section 6 …

# Technical Description

## Overview

The Headwall Hyperspectral Co-Aligned VNIR/SWIR Sensor is designed for airborne or ground based hyperspectral imaging. As the smallest and lightest instrument in its class, the sensor can be purchased as part of a flight-tested, integrated turnkey system with a DJI Matrice 600 Pro UAV and combined with two other instruments that can play crucial roles in ensuring the quality of the collected data[1]:

- A global-positioning system combined with an inertial measurement unit (GPS/IMU), that keeps track of the geographical coordinates during flight and is a key source of data during post-processing of the images.

- A LiDAR, which uses pulsed lasers to measure the distance from the UAV to the ground

Key features of the Headwall Hyperspectral Co-Aligned VNIR/SWIR Sensor with DJI Matrice 600 PRO[1]:

    - Smallest and lightest instrument in its class

    - 400 – 2500 nm wavelength range

    - Dual VNIR & SWIR sensors with co-aligned pixels

    - 640 spatial pixels / 270 VNIR and 260 SWIR spectral pixels

    - Factory data tested and flight ready

    - Integrated high performance GPS/IMU

    - Solid state internal data storage

    - 16-channel LiDAR

Table 1: Technical characteristics of the Hyperspectral Co-Aligned VNIR/SWIR Airborne Sensor with DJI Matrice 600 Pro [1]

| Characteristic | Details |
| --- | --- |
| Spectral range (nm) | VNIR (400-1000) SWIR (900-2500) |
| Spectrograph design | High throughput aberration-corrected concentric imager |
| Spectral pixels | 270 267 |
| Spatial pixels | 640 |
| Detector pixel pitch (μ) | 7.4 15 |
| Dispersion per Pixel (nm/pixel) | 2.2 6 |
| FWHM Slit Image (nm) | 6 10 |
| F/# | 2.5 |
| Slit width (μ) | 20 |
| Slit length (mm) | 6 10.4 |
| Camera technology | CMOS Stirling-cooled MCT |
| Max frame rate (Hz) | 350 200 |
| Bit depth | 12 16 |
| Size (inches / mm) | 10.7 x 8.3 x 6.5 / 272 x 211 x 154 |
| Weight / Weight including LiDAR (kg) | 2.83 / 3.63 |
| Data storage | Internal solid-state drive: 480 GB per sensor |
| IO connectivity | GigE |
| GPS/IMU | Integrated High-Performance model |
| Mounting Options | UAV hard mount / Field Rotary Kit (sold separately) |
| LiDAR (16-channel) | Optional for UAV configurations |
| Power requirement Typical / Max (W) | 26 / 30 |
| Operational temperature range (ºC) | 0- 40 |
| Storage temperature range (ºC) | -30-60 |

Table 2: Technical characteristics of Light weight Real-Time 3D LiDAR Sensor [2]

| Characteristic | Details |
| --- | --- |
| Sensor Time of Flight Distance Measurement with Calibrated Reflectivities Channels Measurement Range Accuracy Returns (Strongest, Last) Field of View (Vertical) Angular Resolution (Vertical) Field of View (Horizontal) Angular Resolution (Horizontal/Azimuth) Rotation Rate Integrated Web Server for Easy Monitoring and Configuration Laser Laser Product Classification Wavelength Beam Size @ Screen Beam Divergence Horizontal Mechanical/Electrical/Operational Power Consumption Operating Voltage Weight Dimensions Shock Vibration Environmental Protection Operating Temperature Storage Temperature Output 3D LiDAR Data Points Generated: 100 Mbps Ethernet Connection UDP Packets C GPS | 16 Up to 100 m ±3 cm (Typical) Single and Dual +15.0° to -15.0° (30°) 2.0° 360° 0.1° – 0.4° 5 Hz – 20 Hz Class 1 Eye-safe per IEC 60825-1:2007 & 2014 903 nm 12.7 mm (Horizontal) x 9.5 mm (Vertical) 0.18° (3.0 mrad); Vertical: 0.07° (1.2 mrad) # 8W # 9 V – 18 V 590 g (without Cabling and Interface Box) 103 mm Diameter x 72 mm Height 500 m/s2 Amplitude, 11 ms Duration 5 Hz to 2,000 Hz, 3 Grms IP67 -10°C to +60°C -40°C to +105°C - Single Return Mode: ~300,000 points per second - Dual Return Mode: ~600,000 points per second - Time of Flight Distance Measurement - Calibrated Reflectivity Measurement - Rotation Angles - Synchronized Time Stamps (μs resolution) GPRMC NMEA Sentence from GPS Receiver (GPS not included) |

Table 3: Technical characteristics of the Trimble AP15 GNSS-Inertial System [3]

| Characteristic | Details |
| --- | --- |
| Advanced Applainix In-Fusion GNSS-Inertial integration Solid-State MEMS IMU with Applanix SmartCall Advanced Trimble Maxwell 6 Custom GNSS survey (two chipsets) 220 channels per chipsets | GPS: simultaneous L1 C/A, L2C, L2E, L5 GLONASS: simultaneous L1 C/A, L2 C/A, L2 P, L3 CDMA BeiDou: B1, B2 Galileo: simultaneous L1 BOC, E5A, E5B, E5AItBOC TZSS: L1 C/A, L1 SAIF, L2C, L5 SBAS: simultaneous L1 C/A, L5 L-Band: OmniSTAR VBS, HP, XP and G2, Trimble CenterPoint RTX |
| High precision multiple correlator for GNSS pseudo range measurements Unfiltered, unsmoothed pseudo range measurements data for low noise, low multipath error, low time domain correlation and high dynamic response Very low noise GNSS carrier phase measurements with <1 mm precision in a 1 Hz bandwidth Proven Trimble low elevation tracking technology Two antenna heading aiding (GNSS Azimuth Measurement System) Support for optional Distance Measurement Indicator Input Support for optional POSPac Mobile Mapping Suite post-processing software No export permit required Size Weight Power Connectors (with connector board) Connectors (with connector board) | 130 L x 100 W x 39 H mm (nominal) 130 L x 100 W x 43 H mm (with optional connector board) 0.28 (nominal), 0.38 kg (with optional connector board) 10-28 Volts DC, 20 Watts (max, with GAMS Option) I/O: Samtec QSH/060-01-L-D-DP-A-RT1 Power: Samtec IPBT-103-H2-X-D-3 |
| Temperature IMU Range Temperature Power Size Weight | -40 degC to +75 degC(Operational) -55 degC +86 degC (Storage) +/- 6g, +/- 350 dps -20 / +55 C +4.5 / -16Vdc, 1 W (max) 43L x 47 W x 12 H mm 0.015 kg |

## Theory of Operation

Combining visible, near infrared (VNIR) and shortwave infrared (SWIR) sensors into one package, the VNIR-SWIR camera from Headwall Photonics can collect wideband hyperspectral data from 400-2500 nm using one instrument. The VNIR sensor is a CMOS image sensor sensitive in the 400 – 1000 nm range, while featuring a pixel pitch of 7.4 µm and a maximum frame rate of 330 fps. The SWIR sensor is a Stirling-cooled MCT detector that is sensitive in the 900-2500 nm range and features a pixel pitch of 15 µm while being able to reach >100 fps. Pixels from each sensor are co-registered through software, making the collected data truly aligned during post-processing. [1]

Headwall’s VNIR-SWIR imaging sensor leverages a patented aberration-corrected design that provides very high spectral and spatial resolution with stable measurement accuracy. The dual hyperspectral sensors do not exhibit image aberrations such as stray light, optical distortions or thermal instabilities. Along with aberration-corrected imaging, the Hyperspectral VNIR-SWIR Sensor package offers a wide field of view and high signal-to-noise performance.

Figure 1: Headwall VNIW-SWIR Co-Aligned + Internal Data Storage + GPS/IMU + LiDAR [1]

The GPS keeps track of the UAV’s location while the IMU accounts for the effects of roll, pitch and yaw experienced during the flight. The GPS collects data on latitude, longitude, and altitude of the craft while aloft. To accurately calculate the size of each pixel in the image as projected on the ground, elevation of the ground relative to the centre of the earth is needed as well. Elevation as function of latitude and longitude can be stored in a Digital Elevation Model (DEM), while LiDAR data can be used to create an accurate DEM. The quality of the point-cloud data captured from the integrated LiDAR

systems differ depending on whether you are using theStandard or High-performance GPS/IMU. In addition to affecting LiDAR data, inaccuracies in GPS/IMU data result in image distortions that impact the ortho-mosaicking of flight-swath images.

Trimble AP15 GNSS-Inertial Sytems[3] is an embedded GNSS-Inertial OEM board set plus Inertial Measurement Uni, in a compact form factor. It is designed to give system integrators the ability to harness the best in GNSS multi-frequency positioning technology, with the superior capabilities of inertial data of continuous mobile positioning in poor signal environments, and for the orientation of imaging sensors.

GNSS key features[3]:

- Centimetre level mobile positioning accuracy

- Industry leading continuous positioning performance in GNSS denied environments

- Full position and orientation solution for direct georeferencing of remote sensing instruments

- High performance GNSS two antenna heading aiding from single receiver

- Solid-state, purpose built-compact MEMS IMU featuring Applanix SmartCall compensation technology.

Figure 2: Trimble AP15 GNSS-inertial system circuit board [3]

Velodyane LiDAR’s Puck LITE [2] is a lighter weight version of the VLP-16 Puck for applications that have lower weight requirements. It has a 360° surround view to capture real-time 3D LiDAR data that includes distance and calibrated reflectivity measurements. It has a range of 100 m width dual return mode to capture greater detail in the 3D image while the power consumption is approximately 8W. A compact footprint and an industry leading weight of 590 g for LiDAR sensor with high resolution makes it ideal for UAV/drone and mobile applications in the areas of 3D mapping/imaging, inspection and navigation.

Figure 3: Puck Lite Velodyne LiDAR [2]

# Calibration History and Uncertainty Budget

## Calibration History

## Uncertainty Budget

# Instrument Operation

## Instrument Setup

## Performing a Measurement

# Care and Storage

###### Appendix

Calibration Report

# Applicable and Reference Documents

[1]	Headwall Photonics, “Hyperspec Co-Aligned VNIR / SWIR Airborne Sensor with DJI Matrice 600 Pro: Product Data Sheet”, Bolton, Massachusetts, 2019.

[2]	Velodyne LiDAR, “Puck LITE, Light Weight Real-Time 3D LiDAR Sensor”, Morgan Hill, CA, 2016.

[3]	Trimble Applanix, “Trimble AP15 Datasheet”, Richmond Hill, Ontario, 2016.